10.

clc;

clear;

close all;

%% Read speech signal

[x, Fs] = audioread('speech.wav');

% Convert stereo to mono

if size(x,2) > 1

x = mean(x,2);

end

%% Add noise

noise = 0.05 * randn(size(x));

noisy = x + noise;

%% STFT parameters

win = hamming(256);

noverlap = 128;

nfft = 512;

%% Spectrogram of noisy speech

[S,F,T] = spectrogram(noisy, win, noverlap, nfft, Fs);

%% Estimate noise spectrum

% First 5 frames are assumed to contain mainly noise

noise_mag = mean(abs(S(:,1:5)),2);

%% Spectral subtraction

magnitude = abs(S);

phase = angle(S);

clean_mag = max(magnitude - noise_mag, 0);

%% Reconstruct spectrum

S_clean = clean_mag .* exp(1j*phase);

%% Inverse STFT

clean = istft(S_clean, Fs, ...

'Window', win, ...

'OverlapLength', noverlap, ...

'FFTLength', nfft);

clean = clean(:);

%% Match signal lengths

L = min(length(x),length(clean));

x = x(1:L);

noisy = noisy(1:L);

clean = clean(1:L);

%% Plot signals

figure;

subplot(3,1,1);

plot(x);

grid on;

xlabel('Sample');

ylabel('Amplitude');

title('Original Speech Signal');

subplot(3,1,2);

plot(noisy);

grid on;

xlabel('Sample');

ylabel('Amplitude');

title('Noisy Speech Signal');

subplot(3,1,3);

plot(clean);

grid on;

xlabel('Sample');

ylabel('Amplitude');

title('Speech After Spectral Subtraction');

%% Play noise-reduced speech

soundsc(clean,Fs);

output:

Amplitude

|

|    /\      /\          /\

|___/  \____/  \___/\____/  \____

|

+--------------------------------> Sample

Amplitude

|

| /\ /\   /\/\ /\  /\/\ /\ /\

|/  \/ \_/    V  \_/    V V  \__

|

+--------------------------------> Sample

Amplitude

|

|    /\      /\          /\

|___/  \____/  \___/\____/  \____

|

+--------------------------------> Sample