4.1

clc;

clear;

close all;

% Specifications

Fs = 1000;          % Sampling frequency

Fc = 200;           % Cut-off frequency

N = 50;             % Filter order

% Normalized cut-off frequency

Wn = Fc/(Fs/2);

% FIR Low-pass filter using Rectangular window

b = fir1(N, Wn, 'low', rectwin(N+1));

% Frequency response

[H, f] = freqz(b, 1, 1024, Fs);

% Plot frequency response

figure;

plot(f, 20*log10(abs(H)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR Low-Pass Filter - Rectangular Window');

xlim([0 Fs/2]);

% Display coefficients

disp('Filter Coefficients:');

disp(b);

output:

- Passband: approximately 0–200 Hz

- Cut-off frequency: 200 Hz

- Stopband begins after the transition region

- Rectangular window has relatively high side lobes.

4.2

clc;

clear;

close all;

% Specifications

Fs = 1000;          % Sampling frequency

Fc = 200;           % Cut-off frequency

N = 50;             % Filter order

% Normalized cut-off frequency

Wn = Fc/(Fs/2);

% FIR Low-pass filter using Hamming window

b = fir1(N, Wn, 'low', hamming(N+1));

% Frequency response

[H, f] = freqz(b, 1, 1024, Fs);

% Plot frequency response

figure;

plot(f, 20*log10(abs(H)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR Low-Pass Filter - Hamming Window');

xlim([0 Fs/2]);

% Display coefficients

disp('Filter Coefficients:');

disp(b);

output:

- Passband: approximately 0–200 Hz

- Hamming window produces lower side lobes than the rectangular window.

- Stopband attenuation is better than the rectangular-window design.

- The transition band is somewhat wider.

4.3

clc;

clear;

close all;

Fs = 1000;

Fc = 200;

N = 50;

Wn = Fc/(Fs/2);

% Design filters

b_rect = fir1(N, Wn, 'low', rectwin(N+1));

b_hamm = fir1(N, Wn, 'low', hamming(N+1));

b_hann = fir1(N, Wn, 'low', hann(N+1));

% Frequency responses

[H1, f] = freqz(b_rect, 1, 1024, Fs);

[H2, ~] = freqz(b_hamm, 1, 1024, Fs);

[H3, ~] = freqz(b_hann, 1, 1024, Fs);

% Plot comparison

figure;

plot(f, 20*log10(abs(H1)));

hold on;

plot(f, 20*log10(abs(H2)));

plot(f, 20*log10(abs(H3)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Comparison of FIR Low-Pass Filters');

legend('Rectangular', 'Hamming', 'Hanning');

xlim([0 500]);

ylim([-100 5]);

output: