5.1

clc;

clear;

close all;

Fs = 1000;

Fc = 300;

N = 50;

Wn = Fc/(Fs/2);

% FIR High-pass filter

b = fir1(N, Wn, 'high', rectwin(N+1));

% Frequency response

[H,f] = freqz(b,1,1024,Fs);

figure;

plot(f,20*log10(abs(H)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR High-Pass Filter - Rectangular Window');

xlim([0 Fs/2]);

disp('Filter Coefficients:');

disp(b);

5.2

clc;

clear;

close all;

Fs = 1000;

Fc = 300;

N = 50;

Wn = Fc/(Fs/2);

% FIR High-pass filter

b = fir1(N, Wn, 'high', hamming(N+1));

% Frequency response

[H,f] = freqz(b,1,1024,Fs);

figure;

plot(f,20*log10(abs(H)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR High-Pass Filter - Hamming Window');

xlim([0 Fs/2]);

disp('Filter Coefficients:');

disp(b);

5.3

clc;

clear;

close all;

Fs = 1000;

Fc = 300;

N = 50;

Wn = Fc/(Fs/2);

% FIR High-pass filter

b = fir1(N, Wn, 'high', hann(N+1));

% Frequency response

[H,f] = freqz(b,1,1024,Fs);

figure;

plot(f,20*log10(abs(H)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR High-Pass Filter - Hanning Window');

xlim([0 Fs/2]);

disp('Filter Coefficients:');

disp(b);

output:

FIR HIGH-PASS FILTERS

0 dB |                    ┌──────────────────

|                   /

|                 /

|_______________/

-40 dB|_______________

+-----------------------------------------

0          300                    500

Frequency (Hz)

Rectangular / Hamming / Hanning