EXP:2.1

clc;

clear;

close all;

% Filter Specifications

Wp = 0.4;      % Passband edge (normalized)

Ws = 0.55;     % Stopband edge (normalized)

Rp = 1;        % Passband ripple (dB)

As = 40;       % Stopband attenuation (dB)

% Calculate filter order and cutoff frequency

[N, Wn] = cheb1ord(Wp, Ws, Rp, As);

% Design Chebyshev Type-I Low-Pass Filter

[b, a] = cheby1(N, Rp, Wn);

% Display filter specifications

fprintf('Filter Order = %d\n', N);

fprintf('Cutoff Frequency = %.2f * pi rad/sample\n', Wn);

% Stability Check

if all(abs(roots(a)) < 1)

disp('System is STABLE');

else

disp('System is UNSTABLE');

end

%% Magnitude and Phase Response

figure;

freqz(b, a);

title('Magnitude and Phase Response');

%% Impulse Response

figure;

impz(b, a);

title('Impulse Response');

%% Group Delay

figure;

grpdelay(b, a);

title('Group Delay');

%% Pole-Zero Plot

figure;

zplane(b, a);

title('Pole-Zero Plot');

%% Step Response (Optional)

figure;

x = ones(1, 50);      % Unit step input

y = filter(b, a, x);

stem(0:49, y, 'filled');

grid on;

xlabel('Samples');

ylabel('Amplitude');

title('Step Response');

OUTPUT:

Filter Order = 6

Cutoff Frequency = 0.40 * pi rad/sample

System is STABLE