12.

clc;

clear;

close all;

% Sampling frequency and cutoff frequency

Fs = 8000;

Fc = 1000;

% FIR filter order

N = 32;

% Normalized cutoff frequency

Wn = Fc/(Fs/2);

% Design FIR Low-Pass Filter using Hamming Window

b = fir1(N, Wn, 'low', hamming(N+1));

% Display filter coefficients

disp('FIR Filter Coefficients:');

disp(b);

% Frequency response

figure;

freqz(b, 1, 1024, Fs);

title('FIR Low-Pass Filter for Audio Signal');

% Read audio signal

[x, Fs_audio] = audioread('audio.wav');

% Convert stereo to mono

if size(x,2) > 1

x = mean(x,2);

end

% FIR filtering

y = filter(b, 1, x);

% Plot original and filtered audio

figure;

subplot(2,1,1);

plot(x);

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('Original Audio Signal');

subplot(2,1,2);

plot(y);

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('Filtered Audio Signal');

% Play filtered audio

soundsc(y, Fs_audio);

% Save filtered audio

audiowrite('filtered_audio.wav', y, Fs_audio);

output: