EXP.2.2

clc;

clear;

close all;

% Specifications

Rp = 1;        % Passband Ripple (dB)

Rs = 40;       % Stopband Attenuation (dB)

Wp = 0.5;      % Passband Edge (Normalized)

Ws = 0.35;     % Stopband Edge (Normalized)

% Find Minimum Filter Order

[n, Wn] = cheb1ord(Wp, Ws, Rp, Rs);

% Design Chebyshev Type-I High-Pass Filter

[b, a] = cheby1(n, Rp, Wn, 'high');

% Display Filter Details

fprintf('Filter Order = %d\n', n);

fprintf('Cutoff Frequency = %.4f\n', Wn);

% Stability Check

if all(abs(roots(a)) < 1)

disp('System is STABLE');

else

disp('System is UNSTABLE');

end

%% Magnitude and Phase Response

figure;

freqz(b, a);

title('Magnitude and Phase Response');

%% Impulse Response

figure;

impz(b, a);

title('Impulse Response');

%% Step Response

figure;

x = ones(1, 50);      % Unit Step Input

y = filter(b, a, x);

stem(0:49, y, 'filled');

grid on;

xlabel('Samples');

ylabel('Amplitude');

title('Step Response');

%% Group Delay

figure;

grpdelay(b, a);

title('Group Delay');

%% Pole-Zero Plot

figure;

zplane(b, a);

title('Pole-Zero Plot');

OUTPUT:

Filter Order = 6

Cutoff Frequency = 0.5000

System is STABLE

/tmp/ipykernel_1394/1391377556.py:69: UserWarning: The filter's denominator is extremely small at frequencies [0.000, 0.002, 0.003, 0.005], around which a singularity may be present

w_gd, gd = signal.group_delay((b, a), w=np.linspace(0, np.pi, 2000)) # Use more points for smooth plot