clc;

clear;

close all;

% Specifications

Fs = 1000;          % Sampling frequency (Hz)

Fp1 = 100;          % Lower passband frequency (Hz)

Fp2 = 300;          % Upper passband frequency (Hz)

Rp = 1;             % Passband ripple for Chebyshev Type-I (dB)

Rs = 40;            % Stopband attenuation (dB)

% Normalized frequencies

Wp = [Fp1 Fp2] / (Fs/2);

Ws = [80 320] / (Fs/2);

%% Butterworth Band-Pass Filter

[Nb, Wnb] = buttord(Wp, Ws, Rp, Rs);

[b_butt, a_butt] = butter(Nb, Wnb, 'bandpass');

%% Chebyshev Type-I Band-Pass Filter

[Nc, Wnc] = cheb1ord(Wp, Ws, Rp, Rs);

[b_cheb, a_cheb] = cheby1(Nc, Rp, Wnc, 'bandpass');

%% Display Filter Orders

fprintf('Butterworth Filter Order = %d\n', Nb);

fprintf('Chebyshev Type-I Filter Order = %d\n', Nc);

%% Frequency Response

[H1, f] = freqz(b_butt, a_butt, 2048, Fs);

[H2, ~] = freqz(b_cheb, a_cheb, 2048, Fs);

figure;

plot(f, 20*log10(abs(H1)));

hold on;

plot(f, 20*log10(abs(H2)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth vs Chebyshev Type-I IIR Band-Pass Filters');

legend('Butterworth', 'Chebyshev Type-I');

xlim([0 500]);

%% Pole-Zero Plot

figure;

subplot(1,2,1);

zplane(b_butt, a_butt);

title('Butterworth Band-Pass Filter');

subplot(1,2,2);

zplane(b_cheb, a_cheb);

title('Chebyshev Type-I Band-Pass Filter');